Serial To Wireless Lan 802.11b/g : E-P132-W

WLAN

- Standard Compliance: IEEE 802.11 b/g

- Spread Spectrum Technology: DSSS, OFDM

- Tx Power 11b: Maximum 20 dBm

- Tx Power 11g: Maximum 18 dBm

- Rx Sensitivity: -70 dBm @ 54 Mbps, -85 dBm @ 11Mbps

- Transmission Rate: 54 Mbps (max.) with auto fallback (54,48, 36, 24, 18, 12, 11, 9, 6, 5.5, 2, 1 Mbps)

- Transmission distance: Up to 100 meters(@12 Mbps, in open areas)

- Security: WEP 64-bit/128-bit data encryption, AES, WPA2

- Antenna Connector: Reverse SMA

- Network Mode: Infrastructure, Ad-Hoc

LAN for configuration / Ethernet hub

- Ethernet: 10/100 Mbps, RJ45

- Protection: Built-in 1.5 KV magnetic isolation

Serial

- No. of ports: 1 RS-232/422/485 port, male DB9, S/W selectable

- RS-232 Signals: TxD, RxD, RTS, CTS, DTR, DSR, DCD, GND

- RS-422 Signal : Tx+ , Tx- , Rx+ , Rx – ( Surge Protect )

- RS-485 Signal : Data+ , Data- ( Surge Protect )

- Serial Line Protection: 15 KV ESD for all signals

Serial Communication Parameters

- Parity: None, Even, Odd, Space, Mark

- Data bits: 5, 6, 7, 8

- Stop bits: 1, 1.5, 2

- Flow control: RTS/CTS, XON/XOFF

- Speed: 110 bps to 460.8+ Kbps

Power Requirements

- Power input 1:  12 VDC

Environmental

- Operating Temperature: -10 to 70°C (32 to 131°F), 5 to 95% RH

Storage Temperature:-20 to 85°C (-4 to 185°F), 5 to 95%RH

Regulatory Approvals

- RoHS

- EMC: FCC Class A, CE Class A

Software Features

- Protocols: ICMP, IP, TCP, UDP, DHCP, BootP, ARP / RARP, Telnet, RTelnet, DNS, SNMP MIB II, HTTP, SMTP, SNTP

- Serial mode: Virtual Com / TCP Server / TCP Client / UDP / Serial Tunnel with advanced settings of

  - TCP Alive Check Timeout

  - Inactivity Timeout

  - Delimiter for Data Packing

  - Force TX Timeout for Data Packing

- 4 Hosts simultaneous connection: Virtual Com / TCP server / TCP Client (only raw data)

- Security: HTTPS, SSH v2

- Event notification by email / SNMP trap

SNMP Trap

- Cold/Warm Start

- DSR, DCD Changed

- IP Changed

- Authentication Fail

SNMP Response

- System MIB,

- Interface MIB,

- ICMP, IP, TCP MIB

- UDP MIB

- RS-232 MIB

- Utilities: Windows NT/2000/XP/2003/VISTA

  - Device discovery

  - Auto IP report

  - Device setting (run-time change, no rebooting)

  - Access control list

  - Group setting

  - Device monitoring

  - Serial port monitoring

  - Log info

  - Group Firmware update batch

- Virtual COM/TTY Drivers (WDM mode, configuration in windows device manager): Windows NT/2000/XP/2003,VISTA,WIN 7